Taken directly from the DIGISCAN manual, 22/4/91 by M.L.C...

3.1. Digiscan file format.

Digiscan uses the same file format that electric paintbrush uses to store

its picture files. All digiscan graphic files have a '.EP' file extention and

may be loaded directly into Electric Paintbrush. This format is identical to

the MacPaint file format.

Each picture is a 576x720 bit image, occupying 720 horizontal scan lines

of 72 bytes. A bit set (1 value) indicates a RESET pixel and a reset bit (0

value) indicates a SET pixel. Thus, the value of each bit in the bit map must

be inverted (ie, logically NOTed) in order to display the picture on a

microbee as white pixels on a black background.

As each picture file would take over 50k of disk space, the bit image data

has been compressed. This reduced the image data to around 10k (rubbish! That

is wrong. A picture can take from 2k to 40k depending on complexity - M.L.C),

although this will vary, depending on the complexity of the bit map - a

completely blank picture contains only 1952 bytes.

The bytes in each scan are encoded into two types of records - mixed data

and repeating data.

A mixed data record consists of a byte indicating the record length (1-72

bytes) and the raw data. This type of record is used when each byte in a scan

line is different from its neighbour. Such records actually take up one more

byte than the original scan line data.

Repeating data records are a much more efficient method of storing data.

Most images contain repeating bit patterns. Repeating data records are

represented by a byte to indicate the number of bytes to repeat and the actual

bit pattern that will be repeated. This method allows you to compress up to

127 bytes of uncompressed data into two bytes.

Combinations of mixed data and repeating records may be stored in the same

file. The eighth bit in the first byte of the first record (known as the

counter byte) identifies the record type. If the eighth bit is set (1 value)

then that record is a repeating record. If the eighth bit is reset (0 value)

then it is a mixed data record.

As well as storing image data, the first 512 bytes of a MacPaint picture

file is a pattern header. The header is used for storing pattern data, but is

not used by either Electric Paintbrush or Digiscan. It has been retained for

file compatibility.

3.1.1. Mixed data records.

A mixed data record consists of a counter byte (indicating the record

length of 1-72 bytes) and the image data.

The eighth bit of the counter byte will always be reset (0 value) with

mixed data records. The least significant 7 bits of the counter byte contain

the number of data bytes in the record. This value starts from zero -

therefore, if the value counter byte is 15 then the following 16 bytes will

form the bit image.

3.1.2. Repeating data records.

A repeating data record is formed by two bytes. The first byte is always

the counter byte while the second byte is always the data byte.

The eighth bit of the counter byte will always be set (1 value) with

repeating data records. The seven least significant bits of the counter byte

contain a value representing the number of bytes that are to be repeated. This

number can be obtained by logically NOTing the counter byte and adding 1 (the

basic and turbo pascal subroutines shown below will return this value from the

count byte). The data byte contains the bit map of the data to be displayed.

procedure GetNum(z:byte);

begin

z:=1-z;

end;

05000 VAR (Z)

05010 Z=256-Z

05020 Z=Z+1

05030 RETURN

Please note that a repeating data record can only repeat a maximum of 72

bytes (ie, one complete line of the picture).

END OF NOTES...

This was taken DIRECTLY and VERBATIMLY (yuk!) out of the (unreleased and

unfinished and unpolished) Digiscan manual. DON'T whinge to me if it doesn't

make sense!!!

M.L.C 8-)

##