Teledit 1.1	   3/20/82

Documentation by Nigel Harrison

[Editor's note: "Teledit" is a cross between the original BDS

Telnet program, Ward Christensen's MODEM program, its MODEM7

derivative, a C version called XMODEM, and Nigel's tiny

screen editor. In fact, this program was used for a while as

the primary editor on a CDC 110 micro system, until they got

Mince up. It may not be the LAST modem program you'll ever

need, but it's gotta have more roots than any other!

-leor]

Teledit is a communications program for transmitting files and

connecting to other systems or networks as an ASCII terminal.  It has a

simple editor with which one can manipulate lines of text collected

during a session with a remote system.  The file transmission modes

allow sending/receiving either binary or text files.  The modes selected

from the menu are:

T: Terminal mode - no text collection

Teledit behaves like an ASCII terminal.  Eight-bit characters are sent

and received; no parity bits are checked, inserted or removed.  To

return to the selection menu the SPECIAL character is typed.  The

The SPECIAL character of <ctrl>shift uparrow was chosen because it

is unlikely to be struck accidentally.  To change the SPECIAL character,

recompile Teledit with the desired #define SPECIAL ...

X: terminal mode with teXt collection

Same as terminal mode above, except that any text characters received

on the communication link are saved in a text buffer.  The tab, newline

and formfeed characters are also placed into the buffer; any other

characters are discarded.  While in terminal mode, the editor is

entered by holding down the control key while typing the letter

"E" (control-E).  X mode prompts the user for a filename to be used

for the gathered text.  After 500 lines of text have been collected

and saved in the text buffer, each additional line collected causes

the console bell (alarm) to sound.  When this happens the user should

find a convenient time to suspend communication with the remote

station so that the accumulated text can be saved (flushed) onto

disk as described below.

G: toGgle echo mode (currently set to echo)

Should not be toggled if the user is communicating in full duplex

mode and receiving an echo from the remote station, or the user is

in half duplex mode. Use this option to talk to another person running

Teledit, typically in between file transfers to inform the person of the

next file to be transmitted.

E: Edit text collected

Enters the editor from the menu display.  This will not work until

X: terminal mode with teXt collection has been entered and a text

collection file opened.  Editor commands are described below.

F: Flush text collection buffer to text collection file

Flushes the text collection buffer accumulated in text collection

mode. Does not close the file.

U: select cp/m User area

For users who have user areas, others should ignore this command.

V: select cp/m driVe

Select any of the disk drives available.  The drive selected becomes

the currently logged disk.

D: print Directory for current drive and user area

S: Send a file, MODEM protocol

Prompts for the name of the file to send, then waits for the receiver

(who must be using this program or one which uses the same protocol)

to "synch up".

Returns to menu after completion, successful or not.

R: Receive a file, MODEM protocol

Prompts for the name of the file to be received, then waits for the

sender to begin transmission.  The sender must be using Teledit or

a program that employs the same MODEM protocol.

Q: Quit

Quits.  If a text file has been accumulated in X mode, the user

is asked whether or not he/she wants it saved.

SPECIAL: sends SPECIAL character to the communication line,

should that ever be necessary.

Editor

The editor prompts with "*" when entered.  The current line usually

appears directly below the prompt.  The editor commands are:

A: Append (Yank in) a file and insert it before the current line.

B: go to the Beginning of the file and display a page. The current line

becomes the first line of the text file.

F: Find line that contains the pattern following the "F". If found, it

becomes the current line.  Search starts forward and wraps around.

I: enters Insert mode.  To leave insert mode, <ctrl>Z is struck, followed

by <cr>.  The escape insert mode sequence should only be entered at

the beginning of a line; lines having an embedded ^Z are lost.

K: Kill the current line

nK Kill n lines.

L: Find line that begins with the pattern following the "L".

O: Overwrite lines of text until <ctrl>Z is struck.  <ctrl>Z should

be struck only at the beginning of a new line.

P: Page forward in text.  -P pages backward.

Q: Quit editor.

Sn<cr>  Sets screen size to n lines.  The default is 22 for 24-line

displays.  S23 should be used for 25-line consoles, and S28

for the Control Data 110.

Z: Go to Zee end of the file.  Convenient, for example, to go to the end

then -P to view last page.

n<cr>  Move forward in the file by n line(s).

<cr>   Move forward in the file by one line.

-n<cr> Move backward in the file by n line(s).

Space Bar  Move backward in the file by one line.

#      print number of text lines in file.

Installation

Teledit must be compiled in BDS C with the following constants special

to the installer's particular environment.

#define HC "s*" 	/* where s is the string necessary to home the cursor

on the user's console screen */

#define CLEARS "s"	/* where s is the string necessary to clear the screen

on the user's console	*/

CLEARS should already be properly defined in "bdscio.h".  In addition to the

above defines (which must be present during compilation for Teledit to work)

it is IMPORTANT to have storage allocation enabled in both the header file

and in deff.crl.  See the BDS C compiler manual if this has not already been

done.  In addition to CLEARS, the bdscio.h constants MDATA, MSTAT, MOMASK,

MIMASK and MAHI must be correctly set.  See the bdscio.h header file for

information on how to do that.

Any of the editor commands not needed can be removed by removing the corresp-

onding case statement from the editor function in the source code, then

recompiling Teledit.  Good luck!

<< END TELEDIT.DOC>>

###############################################